DDTZ v2.5

by C.B. Falconer

edited by George A. Havach

Introduction:

============

DDTZ v2.5 is a  complete replacement  for DDT,  Digital  Research's

famous  Dynamic Debugging Tool,  with improved functionality,  bug ex-

termination, and full Z80 support.  In general,  DDTZ is fully compat-

ible with the original utility, but it has extra and extended commands

and many fewer quirks.  All Z80-specific instructions  can be (dis)as-

sembled, though in Intel rather then Zilog format.  Furthermore,  DDTZ

will correctly trace ('T' and 'U' commands) both 8080 and Z80 instruc-

tions,  depending on which CPU is operating.  On startup,  the program

announces which CPU it is running on.

The program is invoked by typing

ddtz<ret>

or

ddtz [d:]filespec<ret>

In  the second form,  DDTZ will load  the specified file  into  memory

starting at 0100H,  unless it's a .HEX file that sets its own load ad-

dress.  Besides reporting the  NEXT  free address and  the PC (program

counter) after a successful load, DDTZ also shows the number of memory

pages needed for a  SAVE.  Instead of  having to write  all this down,

just use  the 'Q' command  at any time to redisplay these three values

for the current application.

NOTE: loading more code above the NEXT pointer revises these values.

As in DDT,  when a program is loaded above the area holding the 'A'

and 'L' (and now 'K') command code,  these commands are disabled,  and

the extra memory  is released to the user.  Thus,  DDTZ can occupy  as

little as 3K total memory space.  Unlike DDT,  however,  DDTZ will not

overwrite itself or the system on program loads (except .HEX files).

At initialization,  the stack pointer  (SP)  points to  a return to

DDTZ,  just like for the CCP.  Thus,  programs that normally return to

the CCP will be returned to DDTZ.  The 'B' command  reinitializes this

condition.

The intercept vector copies  the BDOS version number,  etc.,  so an

object program  does not know that  DDTZ is running  (except for BIOS-

BDOS vector size). Thus, programs that check the version number should

execute correctly under DDTZ.

All input parameters can now be entered in any of three formats:

(1) hexadecimal (as in DDT),

(2) decimal, by adding a leading '#' character,

(3) ASCII, by enclosing between either single or double quotes;

either one or two characters are allowed.

Leading blanks in command lines and parameters are absorbed. Either

a comma or a (single) space  is  a valid delimiter.   Either uppercase

or lowercase input is accepted.

The default command  (for anything  not otherwise recognizable)  is

'H'. This allows convenient calculation, along with the other features

described below.  So, to convert a number, just enter it!

As in DDT,  the prompt character is '-', and the only error message

is the query ('?'), which generally kicks you back to command mode.

New Commands (Over DDT):

=======================

NOTE: letters in parenthesis, e.g. "(U)", show the equivalent command

for DDTZM version (compatible with MSDOS debug).

@   Sets or shows (with no parameter)  the internally stored "base"

value.  Also used with the 'S' and 'D' commands  as an optional

parameter  (though without the '@')  to display memory  from an

arbitrary base marker (offset). When set to zero (the default),

it does not affect any screen displays.

B   B)egin:   resets  the USER stack pointer  to its initial value,

such that any program that exits by an RET will return to DDTZ.

DDTZ provides  a default stack space  of approximately 24 bytes

for user programs.

C   C)ompare first_address,last_address,against_address:  shows all

the byte differences between two memory areas, in the format

XXXX aa YYYY bb

where  XXXX and YYYY  are the comparative memory addresses, and

aa  and  bb  are the corresponding byte values.  Can be used to

verify  the identity  of two files  by first loading them  into

different memory areas with the 'R' command (see below).

K   K)eep:  stores the modified memory area to disk under the file-

(W)  name  specified by  the 'I' command,  overwriting  the original

file from which it was loaded (the user is queried before doing

so).   By default,  the image of memory from 0100H  through the

"NEXT" value -1 is saved.  "K first_address,last_address" over-

rides this and allows writing ANY memory area to a file. Almost

a necessity for CPM 3.0 (no SAVE!).

W)rite on DDTZM

Q   Q)uery:  redisplays the "NEXT PC SAVE" report at any time.

(X)  eX)amine size on DDTZM.

W   W)here first_address,last_address,value: searches the specified

(S)  memory area for the value (a 16-bit word, not a byte) and shows

the locations of all such.  Very useful for finding  CALL's  or

JMP's to a particular address, etc.

S)earch on DDTZM

Y   Y)our_option parm1,parm2,address: executes an arbitrary routine

at the specified address,  with the BC and DE registers  set to

parm1 and parm2, respectively.

Z   Displays (but does not alter) the Z80's alternate register set,

including the index registers (disabled if running on an 8080).

On Z80's,  automatically included  as the last part of the dis-

play by the 'X' command.

Based (Offset) Displays:

=======================

The 'D' and 'S' commands  can use a stored base value (offset),  as

set by  the '@' command.  The current @ value  may be overridden for a

single execution of these commands by adding the base as an extra par-

ameter  in the command line.  The effect is  to add this value  to the

first/last address and display accordingly. The address listing on the

left becomes  XXXX:YYYY,  where XXXX is the offset address and YYYY is

the actual memory address being displayed.  For example, if you have a

data area  located at  42B7H  and wish to  preserve easy access,  just

enter "@42b7".  Now, "d0,3f" will dump memory starting at 4237H.

Further Changes from DDT:

========================

A   A)ssemble now accepts  the full Z80 as well as 8080 instruction

set, although it expects them in Intel rather than Zilog format

(see notes below  under  the 'L' command).   When in doubt, try

poking in the hexcode (with the 'S' command) and then  L)isting

to see what the (dis)assembler recognizes.

D   D)isplay or D)ump  will accept  an optional third parameter  to

set the base value for a single execution only. Format has been

cleaned up.

H   H)ex_arithmetic  on two values  also shows  their difference in

decimal. With only one value, converts to hexadecimal, decimal,

and ASCII (low-order byte only).

I   I)nput now allows drive specification  (d:...)  and sets up the

(N)  complete command line, including both FCB's (at addresses 005CH

and 006CH).  The tail (stored at 0081H up) is NOT upshifted.

N)ame on DDTZM

L   L)ist now displays the raw hexcode, especially handy when exam-

(U)  ining non-code areas. Intel (8080 style) mnemonics are used, so

some disassembled instructions may look a little strange. E.g.,

the Z80's 'IN B,(C)' and 'OUT (C),B'  become  'INP B' and 'OUTP

B', respectively;  'LD (nnnn),BC' becomes 'SBCD nnnn', 'ADD IX,

BC' becomes 'DADX B', and 'JP (IX)' becomes 'PCIX'.

U)nassemble on DDTZM

R   R)ead  now permits  loading a file into memory  with an offset,

(L)  which  is added  to  the default load address  of 0100H.   When

reading in  a .HEX file  with  a preset bias,  the  'R' command

will not transfer control to an invalid execution point.   Ano-

ther execution of the  'R' command  will reread the input file,

e.g.:

i blah<ret>

r<ret>

...modify the code and generally mess about...

r<ret>

The original file  is reloaded,  and the modifications  are re-

moved.

L)oad on DDTZM.

S   S)ubstitute  or S)et,  like D)isplay,  now accepts  an optional

(E)  second parameter  to set the base value  for a single execution

only.

E)nter on DDTZM

T   T)rap/trace  on termination  now shows  the complete CPU state.

Traps and traces no longer lock up  when  a user RST 7 instruc-

tion is executed.  Tracing of BDOS/BIOS calls  is heavily trun-

cated, avoiding clutter and preventing system crashes.

NOTE:  The UNDOCUMENTED Z80 op-codes are not handled.  Can crash.

X   eX)amine also shows  what two-byte values  the HL and SP regis-

(R)  ters are actually pointing to. On Z80's, displays the alternate

register set.

R)egisters on DDTZM.

NOTE:  Any use of the 'K' or 'R' command  resets the system DMA trans-

fer address to the standard default value of 0080H.

Command Summary:

===============

DDTZ command                                          DDTZM command

============                                          =============

@ (base)

A)ssemble first_address                                  A

B)egin {i.e., initialize stack and return}               B

C)ompare first_address,last_address,against_address      C

D)ump first_address[,last_address[,base]]                D

F)ill first_address,last_address,value                   F

G)o_to [address][,trap1[,trap2]]                         G

H)ex_arithmetic value1(,value2)                          H

I)nput FCBs_command_line                                 N)ame

K)eep [first_address,last_address]                       W)rite

L)ist_code first_address[,last_address]                  U)nassemble

M)ove first_address,last_address,destination             M

Q)uery {i.e. display memory parameters for application}  X)amine

R)ead_file (offset)                                      L)oad

S)ubstitute_in_memory first_address[,base]               E)nter

T)race_execution [count]                                 T

U)ntrace_execution [count] (i.e. do count instr)     (NOT AVAILABLE)

W)here_is first_address,last_address,word                S)earch

X)amine/change [register|flag]                           R)egister

Y)our_option BC:=parm1,DE:=parm2,call_address            Y

Z)80_register_display                                    Z

( on DDTZM only --> )        Q)uit

P