Usage:

hex2bin <options> [<flags> <filename.hex>]+

bin2hex <options> <flags> <filename.bin>

Options:

-g  --  Use Intel seGmented addressing in hex files for

ROMs between 64k and 1M in size.  Normally, linear

addressing is used for ROMs bigger than 64k.  For

ROMs larger than 1M, this switch has no effect,

since only linear addressing is possible.

-o <output filename> -- default is 'out.bin' or 'out.hex'

No suffix is added to the filename, so the full

file name must be specified.

-p <pad byte> -- default is 0xFF, which is the erased

value for most ROM's.  Pad bytes are not explicitly

written out when 'bin2hex' is used.  This reduces

the size of sparse ROM images.

-R <ROM size> -- may be any value.  Normally this will be

specified as '128k' or 0x10000.  The suffixes 'k'

and 'M' are recognized.  The default value is 64k

for 'hex2bin' and 1M for 'bin2hex'.

-v [<verbosity level>] -- used primarily for debugging.  A

verbosity level of 3 will print each line as it is

processed.

Options are global, and may be specified anywhere on the command

line.  Only 'flags', below, are position sensitive.  Flags must

be specified immediately before the file name they are to affect.

They do not affect more that a single file.  They reset to their

default values before the next input file, scanning left to

right, is processed.

Flags:

Flag values apply only to the following file, and are reset

to the default values before the next file is processed.  The

source and destination flags allow code to be relocated

within the ROM so that code may be loaded at a different

location than where it will ultimately be run.

-d <destination address in output file> -- default is 0.

data will only be loaded between the limits of

the -d <dest> address and the -D <limit> address

minus 1.

-D <destination limit in output file> -- default is 16M.  No data

will be loaded at or above this address.

-s <source address in input file> -- default is 0.  Hex input

is only processed if it falls at addresses in the

.hex input file between the -s <srs> address and the

-S <limit> address minus 1.  Likewise, Bin input

is only processed if it falls between similar limits

in the input ROM image.

-S <source limit in hex file> -- default is 16M.  No input

data at or above this address will be processed.

Numeric values on the command line may be specified in any C-like

syntax:  0x0000 is hexadecimal, 1234 is decimal, and 0177 is

octal.  In addition the suffixes 'k' for kilo- and 'M' for mega-

cause the preceding values to be multiplied by 1024 or 1048576,

respectively.

Examples:

hex2bin -p 0xFF -R32k cpm22.hex -o cpmtest.bin

hex2bin -o cpmtest.bin  cpm22.hex -R 0x8000 -p255

The preceding two lines have identical effects in all

respects.

hex2bin -R 1M  -s 0xd000 -d 0x0800 -D0x2800 image.hex \

pagezero.hex -oROMIMAGE.bin

The above line loads the 'pagezero.hex' file at the exact

addresses specified in the file; however, the 'image.hex' file

is assembled at 0xD000, must run at 0xD000, but is loaded

between 0x800 <= addr < 0x2800.  It is presumed that the

code will be relocated to the correct address before it is

executed.

bin2hex -R128k romimage.bin -o image.hex

Simple conversion from a BIN file to a HEX file.  The ROM

data is limited to 128k.

bin2hex -s 0x1000 -S 0x2000 -d 0x21000 romimage.bin \

-o Relocated.hex

Extract from a ROM image file all of the data between 0x1000

and 0x1FFF, inclusive, and write to an Intel hex file for

re-loading two 64k segments higher in a future ROM.

(end)