7 March 09

## 1.0 Introduction

This version of the BIOS documentation is based on the smaller N8VEM monitor and the MAKROM EPROM image generator program.  I continue to try to document the BIOS as I have modified it in this document.

The N8VEM SBC is based on the Z80 microprocessor.  Though designed in the late 1970’s, this processor is still manufactured today.  Commercial development tools are also widely available for the Z80.  The CP/M-80 operating system, which will run on a z80 processor, has been released into the public domain.  Thus, all the parts required to create a fully functional single board computer are available.  This has allowed the construction of the N8VEM SBC which will boot the CP/M operating system in less than a second!

The N8VEM SBC is designed with an EPROM for booting and as a ROM disk.  It also includes a static RAM chip for processor memory and as a RAM disk.  The ROM and RAM are divided into pages of 32 kB each.  The upper Z80 memory (08000H-0FFFFH) is always the top RAM page and always available to the processor.  The other pages or RAM or ROM can be selected to appear in the lower Z80 memory (00000H-07FFFH).

The N8VEM SBC uses an 82C50, 16C450 or 16C550 UART to provide serial communications to a user console.  It uses an 82C55 to provide parallel I/O which can be used for keyboard and LCD display interface.

The mini-N8VEM SBC provides a battery backed up RAM so drive A retains data when power is removed from the SBC.

The regular N8VEM SBC has a battery backed up clock included and provides interface to the DIN backplane connector for connection to expansion boards.

To configure the CP/M-80 operating system for the N8VEM SBC a custom Basic Input/Output System (BIOS) had to be developed.  This document will detail the programming required to support each subsystem of the N8VEM SBC hardware.  The N8VEM SBC can be broken into the following major subsystems:

- Reset

- Page swapping

- Interrupts

- UART (8250/16450/16550)

- PIO (8255)

- Disk Subsystem

  - ROM

  - RAM

  - FDC (provided on the FDC expansion board)

  - HDC (provided on the FDC expansion board)

The BIOS code can be broken into sections to handle each of these different subsystems, and discussed in detail.

## 2.0 Reset Subsystem

### 2.1 Reset

On reset, the page swapping registers are set to all zeros.  This selects page 0 of the ROM and places it into the lower Z80 memory map.  The Z80 begins operation by executing the instructions beginning at address 0.  This is the boot loader file in the source code.

It performs the following steps:

- Disable interrupts

- Sets the stack pointer to the top of memory

- Copies the monitor image into RAM starting at 0F800H

- Copies the CP/M image into RAM starting at 0D400H

- It also initializes the RAM disk DPB on RAM page 1, offset 080H

- Jumps to the BIOS cold boot address (BOOT)

The first 5 tracks of the ROM (All EPROMs) and RAM disk are reserved to hold the boot system.  The boot image is laid out as follows:

| Track | Sector | Page | Offset | CP/M Module Name | Logical Offset | Load Address |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0000H | Boot Loader | 0000H |  |
| 0 | 1 | 0 | 0080H | DPB | 0080H |  |
| 0 | 2 | 0 | 0100H | First Sector of Monitor | 0100H | FC00H |
| 1 | 1 | 0 | 0480H | Last Sector of Monitor (400H Bytes) | 0480H | FE80H |
| 1 | 3 | 1 | 0100H | First Sector of CCP | 0500H | D400H |
| 2 | 1 | 2 | 0080H | Last Sector of CCP (800H Bytes) | 0C80H | DB80H |
| 2 | 2 | 2 | 0100H | First Sector of BDOS | 0D00H | DC00H |
| 3 | 13 | 3 | 0680H | Last Sector of BDOS (E00H Bytes) | 1A80H | E980H |
| 3 | 14 | 3 | 0700H | First Sector of BIOS | 1B00H | EA00H |
| 4 | 15 | 4 | 0780H | Last Sector of BIOS (Up To D00H Bytes) | 2780H | F680H |
| 5 | 0 | 5 | 0000H | First Sector of Block 0 | 2800H |  |

Table 1 – Boot Image Layout

In this version of the system, the monitor had a lot of text removed, including the help message (Sorry James!).  This reduced the monitor image from nearly 800 hex bytes to about 300 hex bytes.  The new load point is 0FC00h.  This freed 400 hex bytes for a larger BIOS.  This all assumes we stay with the 60K CPM image (0D400h CCP start address).  With a 400 hex byte monitor and a small BIOS, we could build a 63K CPM system.

The RAM disk is laid out identically, except the page has 1 added to the value, and the logical offset has 8000H added to the value to exclude the lower 32K bytes of memory.  This was done to align the RAM and ROM disk geometries so ROM disk images could be tested in RAM.

The BIOS boot code is broken into the cold boot, warm boot and common code.  It provides two BIOS entry points, BOOT and WBOOT.

### 2.2 Cold Boot (BIOS BOOT Entry)

The cold boot entry is called after the entire operating system is loaded from the disk drive. 	It then turns off the ROM and brings page zero of the RAM.  It sets the initial value of IOBYTE and CDISK, and sets WRMFLG for a cold boot.  The Stack Pointer is set to 080H. It calls the routines to initialize the UART and the PIO.  It outputs the cold boot message and goes to the common boot code (GOCPM).

### 2.3 Warm Boot (BIOS WBOOT Entry)

The warm boot entry is called whenever a transient program completes or when a ctl-c is typed.  It loads the CCP and BDOS image from the boot drive into memory.  It then turns off the ROM and brings page zero of the RAM.  The Stack Pointer is set to 080H.  WRMFLG is set for a warm boot.  It outputs the warm boot message and goes to the common boot code (GOCPM).

### 2.4 Common to Warm and Cold Boot

The common code is the first pace that interrupts are configured and enabled.  It sets the initial values of the vectors in the bottom of RAM for CP/M.  Default DMA is set for the default CP/M buffer at 00080H.  It then gets the value of the CDISK to logon and enters the CCP at either CCP+0 to run the program named in the CCP buffer or CCP+3 to clear the CCP buffer.  CP/M begins takes over operation.

## 3.0 Page Swapping Subsystem

The N8VEM SBC uses two 8-bit registers to select which page of ROM or RAM is available in the bottom 32kB of the Z80 memory map.  The ROM register uses 5 bits to select one of 32 pages of the ROM and one bit to disable the mapping of ROM at all so the RAM will be mapped. The RAM register uses 4 bits to select one of 16 pages of the RAM.  The upper 32kB of the Z80 memory always contains page 15 of the RAM.

One must ensure the following before any page swapping occurs:

- Stack pointer is in the upper 32kB of memory.

- Program is running in the upper 32kB of memory.

- Interrupts are disabled.

Failure to observe these rules may cause the processor to run away (loose context).

FIX01 in the BIOS source code is there to allow calls directly to the BIOS entries from the low TPA without causing a loss of context.  The BIOS entries should not be called directly from user programs, but I used them in programs to test the disks.

## 4.0 Interrupt Subsystem

Interrupts are currently not used in this version of the BIOS.  They are enabled and set to mode 1.  User beware!

## 5.0 Universal Asynchronous Receiver Transmitter (UART) Subsystem

The UART subsystem provides console connection for CP/M.

The BIOS UART code is broken into the UART initialization, console status, console input and console output code.  It provides three BIOS entry points, CONST, CONIN, and CONOUT.  UARTINI is called from the cold boot BIOS routine.

### 5.1 Console Initialization

Currently, it is initialized in the Monitor code.  Initialization code has to be added in BIOS in the empty routine UARTINI to utilize CP/M without the Monitor.

### 5.2 Console Status (BIOS CONST Entry)

The console status entry returns either 0 for no character received or 0FFH for a character ready in register A.  No other registers are used.

### 5.3 Console Input (BIOS CONIN Entry)

The console input entry returns the next character from the console port in register A with the parity bit reset.  No other registers are used.

### 5.4 Console Output (BIOS CONOUT Entry)

The console output entry sends the character in register C to the console port.  Register A is also used.

## 6.0 Parallel I/O Subsystem

Parallel I/O is not configured currently in this version of the BIOS.

The Parallel I/O subsystem provides a printer connection for CP/M.

The BIOS PIO code is broken into the PIO initialization, list status, and list output code.  It provides tw0 BIOS entry points, LISTST and LISTOUT.  PIOINI is called from the cold boot BIOS routine.

### 6.1 PIO Initialization

Currently, it is initialized in the Monitor code.  Initialization code has to be added in BIOS in the empty routine PIOINI to utilize CP/M without the Monitor.

### 6.2 List Status (BIOS LISTST Entry)

The list status entry returns either 0 for ready for a character or 0FFH for not ready for a character.  No other registers are used.

### 6.3 List Output (BIOS LISTOUT Entry)

The list output entry sends the character in register C to the printer port.  Register A is also used.

## 7.0 CP/M 2.2 Disk Configuration

In CP/M-2.2, the disk’s configuration and operational data is captured in two tables: the Disk Parameter Header (DPH) and the Disk Parameter Block (DPB).  The DPH and DPB can be configured dynamically, during disk selection.  They will be discussed in detail in the next two sections.  The information here was taken from the CPM 2.0 System Alteration Guide.

### 7.1 Disk Parameter Header

There is one DPH for each drive.  The DPH contains the addresses of the sector translation table, the directory buffer (shared by all drives), the DPB, the check vector for this drive, the allocation vector for this drive and a small scratch storage area.  The DPH address is returned to BDOS from the BIOS SELDSK entry.

The sector translation table is used to provide skewing of sectors on rotating media.  This skewing allows logical sectors to be mapped to physical sectors that are separated by several sectors to speed up reading and writing.  The table can be either byte- or word-wide, depending if block numbers are larger than 255.  It is not used on the ROM or RAM disks so this field is set to zero. On a standard 8-inch single-density (SD) disk, the sectors are skewed 6 sectors apart.   It will be used and discussed in the sections on rotating media – floppy drives and hard drives.

The 128-byte directory buffer is shared by all the drives and is used by BDOS to store the current directory sector on the current drive.

The DPB contains the address of the DPB that describes this disk’s geometry.  The DPB will be discussed in detail in the next section.

The check vector contains entries for each checked directory entry.  The check vectors are not shared between drives. The check mechanism is used to detect changed media.  This would not be used with the RAM, ROM, or hard drives.  It is used with the floppies and any type of removable media, like a PCMCIA card.  The check vector is CKS in size, where CKS = (DRM+1)/4 for removable and 0 for non-removable media.  The check vector can be in uninitialized memory.

The allocation vector contains a bit entry for each block on the drive.  The allocation vectors are not shared between drives. This is used to find unallocated blocks to store new data on the drive.  Logging onto a drive causes BDOS to scan the entire directory to see which blocks are allocated and set that bit in the allocation vector to 1.  The allocation vector is (DSM+1)/8 bytes in length. The allocation vector can be in uninitialized memory.

### 7.2 Disk Parameter Block

The DPB sets the geometry of a particular disk.  One DPB may be shared between several disks that have identical geometry.  There will be different DPBs for each of the ROM types used for the B drive because the drive size varies with the ROM type.  Their values are captured along with some implied values and some DPH values in Table 2.

| Device | 27C256 | 27C512 | 27C010 | 27C020 | 27C040 | 27C080 | RAM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Size | 32K | 64K | 128K | 256K | 512K | 1024K | 512K |
| SPT | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| BSH | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| BLM | 7 | 7 | 7 | 7 | 15 | 15 | 15 |
| BLS | 1024 | 1024 | 1024 | 1024 | 2048 | 2048 | 2048 |
| EXM | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| DSM | 21 | 53 | 117 | 245 | 250 | 506 | 219 |
| DRM | 31 | 63 | 127 | 255 | 255 | 511 | 255 |
| AL0 | 80H | C0H | F0H | FFH | F0H | FFH | F0H |
| AL1 | 00H | 00H | 00H | 00H | 00H | 00H | 00H |
| OFF | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| ALV | 3 | 7 | 15 | 31 | 32 | 25 | 28 |
| TPD | 16 | 32 | 64 | 128 | 256 | 512 | 256 |

Notes:

- RAM has the top and bottom 32KB reserved plus 5 tracks for a total of 5 BLS units. Top is knocked out in the size and bottom is protected by adding 1 to the page calculated for the RAM.

- ROM has 5 tracks reserved for a total of 10 BLS units.

- Directory size is set to allow the disk to contain as many files as there are blocks.

Table 2 – The N8VEM DPB values for various devices.

The Sectors Per Track (SPT) value was chosen to minimize waste in defining the reserved tracks for the loader, monitor and operating system.  SPT was set to 16 for both ROM and RAM disks.

The Block Shift Factor (BSH) and Block Shift Mask (BLM) determine the Block Size (BLS) in accordance with table 3.  = (128 << BSH). I chose a BLS value of 1024 bytes for the ROM and RAM disks.  After tracking down a bug in the RAM disk, I looked at table 3 and saw I was using an illegal BLS for the larger disks. I left BLS set for 1024 bytes for the smaller disks (27C256 to 27C020).  I had to use a BLS of 2048 bytes for the largest disks (27C040, 27C080 and RAM disks).

| BLS | BSH | BLM |
| --- | --- | --- |
| 1024 | 3 | 7 |
| 2048 | 4 | 15 |
| 4096 | 5 | 31 |
| 8192 | 6 | 63 |
| 16384 | 7 | 127 |

Table 3 – BLS definition based on BSH and BLM.

The Extent Mask (EXM) is set based on the block size (BLS) and the Disk Size (DSM) in accordance with table 4.  The N/A is an illegal selection that causes the BDOS to trash the disk.

| BLS | DSM < 256 | DSM > 255 |
| --- | --- | --- |
| 1024 | 0 | N/A |
| 2048 | 1 | 0 |
| 4096 | 3 | 1 |
| 8192 | 7 | 3 |
| 16384 | 15 | 7 |

Table 4 – EXM definition based on BLS and DSM.

The Disk Storage Mask (DSM) is the number of the last valid block on the drive.  This defines the size of the drive less one block in BLS units.  This value does not include the blocks in the reserved tracks (OFF).

The Directory Mask (DRM) is the number of directory entries on the disk minus 1.  On the 27C256 EPROM there are more directory entries than blocks but the minimum number of directory entries is one block.  I attempted to maintain as many directory entries as there are blocks on the drive.  This allows the maximum number of small files on the drive.  Larger files use fewer directory entries, so directory entries will not be the limiting factor, disk size will be.

## 8.0 Read Only Memory Subsystem

The ROM subsystem provides nonvolatile memory to boot the system and provide a minimum subset of CP/M utilities to load other utilities into the RAM disk.  As the ROMs get larger, more CP/M utilities can be provided in the ROM.

Let us look at the ROM disk geometries that I chose for the various EPROMs types.

### 8.1 27C256

The 27C256 EPROM is configured IAW table 1. The 32kB ROM is laid out as follows:

| Track | Use | Page | Offset | Block |
| --- | --- | --- | --- | --- |
| 15 | Data | 0 | 07800-07FFFH | 20/21 |
| 14 | Data | 0 | 07000-077FFH | 18/19 |
| 13 | Data | 0 | 06800-06FFFH | 16/17 |
| 12 | Data | 0 | 06000-067FFH | 14/15 |
| 11 | Data | 0 | 05800-05FFFH | 12/13 |
| 10 | Data | 0 | 05000-057FFH | 10/11 |
| 09 | Data | 0 | 04800-04FFFH | 08/09 |
| 08 | Data | 0 | 04000-047FFH | 06/07 |
| 07 | Data | 0 | 03800-03FFFH | 04/05 |
| 06 | Data | 0 | 03000-037FFH | 02/03 |
| 05 | Dir & Data | 0 | 02800-02FFFH | 00/01 |
| 04 | CPM | 0 | 02000-027FFH |  |
| 03 | CPM | 0 | 01800-01FFFH |  |
| 02 | CPM | 0 | 01000-017FFH |  |
| 01 | Monitor & CPM | 0 | 00800-00FFFH |  |
| 00 | Loader , DPB & Monitor | 0 | 00000-007FFH |  |

Table 5 – 27C256 EPROM Geometry

### 8.2 27C512

The 27C512 EPROM is configured IAW table 1.  The 64kB ROM is laid out as follows:

| Track | Use | Page | Offset | Block |
| --- | --- | --- | --- | --- |
| 31 | Data | 1 | 07800-07FFFH | 52/53 |
| 30 | Data | 1 | 07000-077FFH | 50/51 |
| … | … | … | … | … |
| 17 | Data | 1 | 00800-00FFFH | 24/25 |
| 16 | Data | 1 | 00000-007FFH | 22/23 |
| 15 | Data | 0 | 07800-07FFFH | 20/21 |
| 14 | Data | 0 | 07000-077FFH | 18/19 |
| … | … | … | … | … |
| 07 | Data | 0 | 03800-03FFFH | 04/05 |
| 06 | Data | 0 | 03000-037FFH | 02/03 |
| 05 | Dir | 0 | 02800-02FFFH | 00/01 |
| 04 | CPM | 0 | 02000-027FFH |  |
| 03 | CPM | 0 | 01800-01FFFH |  |
| 02 | CPM | 0 | 01000-017FFH |  |
| 01 | Monitor & CPM | 0 | 00800-00FFFH |  |
| 00 | Loader , DPB & Monitor | 0 | 00000-007FFH |  |

Table 6 – 27C512 EPROM Geometry

### 8.3 27C010

The 27C010 EPROM is configured IAW table 1.  The 128kB ROM is laid out as follows:

| Track | Use | Page | Offset | Block |
| --- | --- | --- | --- | --- |
| 63 | Data | 3 | 07800-07FFFH | 116/117 |
| 62 | Data | 3 | 07000-077FFH | 114/115 |
| … | … | … | … | … |
| 49 | Data | 3 | 00800-00FFFH | 88/89 |
| 48 | Data | 3 | 00000-007FFH | 86/87 |
| 47 | Data | 2 | 07800-07FFFH | 84/85 |
| 46 | Data | 2 | 07000-077FFH | 82.83 |
| … | … | … | … | … |
| 33 | Data | 2 | 00800-00FFFH | 56/57 |
| 32 | Data | 2 | 00000-007FFH | 54/55 |
| 31 | Data | 1 | 07800-07FFFH | 52/53 |
| 30 | Data | 1 | 07000-077FFH | 50/51 |
| … | … | … | … | … |
| 17 | Data | 1 | 00800-00FFFH | 24/25 |
| 16 | Data | 1 | 00000-007FFH | 22/23 |
| 15 | Data | 0 | 07800-07FFFH | 20/21 |
| 14 | Data | 0 | 07000-077FFH | 18/19 |
| … | … | … | … | … |
| 07 | Data | 0 | 03800-03FFFH | 04/05 |
| 06 | Dir | 0 | 03000-037FFH | 02/03 |
| 05 | Dir | 0 | 02800-02FFFH | 00/01 |
| 04 | CPM | 0 | 02000-027FFH |  |
| 03 | CPM | 0 | 01800-01FFFH |  |
| 02 | CPM | 0 | 01000-017FFH |  |
| 01 | Monitor & CPM | 0 | 00800-00FFFH |  |
| 00 | Loader , DPB & Monitor | 0 | 00000-007FFH |  |

Table 7 – 27C010 EPROM Geometry

### 8.4 27C020

The 27C020 EPROM is configured IAW table 1.  The 256kB ROM is laid out as follows:

| Track | Use | Page | Offset | Block |
| --- | --- | --- | --- | --- |
| 127 | Data | 7 | 07800-07FFFH | 244/245 |
| 126 | Data | 7 | 07000-077FFH | 242/243 |
| … | … | … | … | … |
| 65 | Data | 4 | 00800-00FFFH | 120/121 |
| 64 | Data | 4 | 00000-007FFH | 118/119 |
| 63 | Data | 3 | 07800-07FFFH | 116/117 |
| 62 | Data | 3 | 07000-077FFH | 114/115 |
| … | … | … | … | … |
| 49 | Data | 3 | 00800-00FFFH | 88/89 |
| 48 | Data | 3 | 00000-007FFH | 86/87 |
| 47 | Data | 2 | 07800-07FFFH | 84/85 |
| 46 | Data | 2 | 07000-077FFH | 82.83 |
| … | … | … | … | … |
| 33 | Data | 2 | 00800-00FFFH | 56/57 |
| 32 | Data | 2 | 00000-007FFH | 54/55 |
| 31 | Data | 1 | 07800-07FFFH | 52/53 |
| 30 | Data | 1 | 07000-077FFH | 50/51 |
| … | … | … | … | … |
| 17 | Data | 1 | 00800-00FFFH | 24/25 |
| 16 | Data | 1 | 00000-007FFH | 22/23 |
| 15 | Data | 0 | 07800-07FFFH | 20/21 |
| 14 | Data | 0 | 07000-077FFH | 18/19 |
| … | … | … | … | … |
| 09 | Data | 0 | 04800-047FFH | 08/09 |
| 08 | Dir | 0 | 04000-047FFH | 06/07 |
| 07 | Dir | 0 | 03800-03FFFH | 04/05 |
| 06 | Dir | 0 | 03000-037FFH | 02/03 |
| 05 | Dir | 0 | 02800-02FFFH | 00/01 |
| 04 | CPM | 0 | 02000-027FFH |  |
| 03 | CPM | 0 | 01800-01FFFH |  |
| 02 | CPM | 0 | 01000-017FFH |  |
| 01 | Monitor & CPM | 0 | 00800-00FFFH |  |
| 00 | Loader , DPB & Monitor | 0 | 00000-007FFH |  |

Table 8 – 27C020 EPROM Geometry

### 8.5 27C040

The 27C040 EPROM is configured IAW table 1.  The 512kB ROM is laid out as follows:

| Track | Use | Page | Offset | Block |
| --- | --- | --- | --- | --- |
| 255 | Data | 15 | 07800-07FFFH | 250 |
| 254 | Data | 15 | 07000-077FFH | 249 |
| … | … | … | … | … |
| 65 | Data | 4 | 00800-00FFFH | 60 |
| 64 | Data | 4 | 00000-007FFH | 59 |
| 63 | Data | 3 | 07800-07FFFH | 58 |
| 62 | Data | 3 | 07000-077FFH | 57 |
| … | … | … | … | … |
| 49 | Data | 3 | 00800-00FFFH | 44 |
| 48 | Data | 3 | 00000-007FFH | 43 |
| 47 | Data | 2 | 07800-07FFFH | 42 |
| 46 | Data | 2 | 07000-077FFH | 41 |
| … | … | … | … | … |
| 33 | Data | 2 | 00800-00FFFH | 28 |
| 32 | Data | 2 | 00000-007FFH | 27 |
| 31 | Data | 1 | 07800-07FFFH | 26 |
| 30 | Data | 1 | 07000-077FFH | 25 |
| … | … | … | … | … |
| 17 | Data | 1 | 00800-00FFFH | 12 |
| 16 | Data | 1 | 00000-007FFH | 11 |
| 15 | Data | 0 | 07800-07FFFH | 10 |
| 14 | Data | 0 | 07000-077FFH | 09 |
| … | … | … | … | … |
| 09 | Data | 0 | 04800-04FFFH | 04 |
| 08 | Dir | 0 | 04000-047FFH | 03 |
| 07 | Dir | 0 | 03800-03FFFH | 02 |
| 06 | Dir | 0 | 03000-037FFH | 01 |
| 05 | Dir | 0 | 02800-02FFFH | 00 |
| 04 | CPM | 0 | 02000-027FFH |  |
| 03 | CPM | 0 | 01800-01FFFH |  |
| 02 | CPM | 0 | 01000-017FFH |  |
| 01 | Monitor & CPM | 0 | 00800-00FFFH |  |
| 00 | Loader , DPB & Monitor | 0 | 00000-007FFH |  |

Table 9 – 27C040 EPROM Geometry

### 8.6 27C080

The 27C080 EPROM is configured IAW table 1.  The 1024kB ROM is laid out as follows:

| Track | Use | Page | Offset | Block |
| --- | --- | --- | --- | --- |
| 511 | Data | 31 | 07800-07FFFH | 506 |
| 510 | Data | 31 | 07000-077FFH | 505 |
| … | … | … | … | … |
| 65 | Data | 4 | 00800-00FFFH | 60 |
| 64 | Data | 4 | 00000-007FFH | 59 |
| 63 | Data | 3 | 07800-07FFFH | 58 |
| 62 | Data | 3 | 07000-077FFH | 57 |
| … | … | … | … | … |
| 49 | Data | 3 | 00800-00FFFH | 44 |
| 48 | Data | 3 | 00000-007FFH | 43 |
| 47 | Data | 2 | 07800-07FFFH | 42 |
| 46 | Data | 2 | 07000-077FFH | 41 |
| … | … | … | … | … |
| 33 | Data | 2 | 00800-00FFFH | 28 |
| 32 | Data | 2 | 00000-007FFH | 27 |
| 31 | Data | 1 | 07800-07FFFH | 26 |
| 30 | Data | 1 | 07000-077FFH | 25 |
| … | … | … | … | … |
| 17 | Data | 1 | 00800-00FFFH | 12 |
| 16 | Data | 1 | 00000-007FFH | 11 |
| 15 | Data | 0 | 07800-07FFFH | 10 |
| 14 | Data | 0 | 07000-077FFH | 09 |
| … | … | … | … | … |
| 13 | Data | 0 | 04800-04FFFH | 08 |
| 12 | Dir | 0 | 04000-047FFH | 07 |
| … | … | … | … | … |
| 06 | Dir | 0 | 03000-037FFH | 01 |
| 05 | Dir | 0 | 02800-02FFFH | 00 |
| 04 | CPM | 0 | 02000-027FFH |  |
| 03 | CPM | 0 | 01800-01FFFH |  |
| 02 | CPM | 0 | 01000-017FFH |  |
| 01 | Monitor & CPM | 0 | 00800-00FFFH |  |
| 00 | Loader , DPB & Monitor | 0 | 00000-007FFH |  |

Table 10 – 27C080 EPROM Geometry

## 9.0 Static Random Access Memory Subsystem

The RAM subsystem provides volatile memory to boot the CP/M system. The top and bottom page of the RAM is used to provide the RAM for the Z80.  The rest of the RAM is used as a RAM disk. Lets look at the RAM disk geometry that I chose for the system.

| Track | Use | Page | Offset | Block |
| --- | --- | --- | --- | --- |
| 224 | Data | 14 | 07800-07FFFH | 219 |
| 223 | Data | 14 | 07000-077FFH | 218 |
| … | … | … | … | … |
| 65 | Data | 5 | 00800-00FFFH | 60 |
| 64 | Data | 5 | 00000-007FFH | 59 |
| 63 | Data | 4 | 07800-07FFFH | 58 |
| 62 | Data | 4 | 07000-077FFH | 57 |
| … | … | … | … | … |
| 49 | Data | 4 | 00800-00FFFH | 45 |
| 48 | Data | 4 | 00000-007FFH | 44 |
| 47 | Data | 3 | 07800-07FFFH | 43 |
| 46 | Data | 3 | 07000-077FFH | 42 |
| … | … | … | … | … |
| 33 | Data | 3 | 00800-00FFFH | 28 |
| 32 | Data | 3 | 00000-007FFH | 27 |
| 31 | Data | 2 | 07800-07FFFH | 26 |
| 30 | Data | 2 | 07000-077FFH | 25 |
| … | … | … | … | … |
| 17 | Data | 2 | 00800-00FFFH | 12 |
| 16 | Data | 2 | 00000-007FFH | 11 |
| 15 | Data | 1 | 07800-07FFFH | 10 |
| 14 | Data | 1 | 07000-077FFH | 09 |
| 13 | Data | 1 | 06800-06FFFH | 08 |
| 12 | Data | 1 | 06000-067FFH | 07 |
| 11 | Data | 1 | 05800-05FFFH | 06 |
| 10 | Data | 1 | 05000-057FFH | 05 |
| 9 | Data | 1 | 04800-04FFFH | 04 |
| 8 | Dir | 1 | 04000-047FFH | 03 |
| 7 | Dir | 1 | 03800-03FFFH | 02 |
| 6 | Dir | 1 | 03000-037FFH | 01 |
| 5 | Dir | 1 | 02800-02FFFH | 00 |
| 4 | CPM | 1 | 02000-027FFH |  |
| 3 | CPM | 1 | 01800-01FFFH |  |
| 2 | CPM | 1 | 01000-017FFH |  |
| 1 | Monitor & CPM | 1 | 00800-00FFFH |  |
| 0 | Loader , DPB & Monitor | 1 | 00000-007FFH |  |

Table 11 – 628512 SRAM Geometry

## 10.0 Floppy Disk Controller Subsystem

Not supported in this version of BIOS.

## 11.0 Hard Disk Controller Subsystem

Not supported in this version of BIOS.